Theft-Alarm LoRaWAN Node

Firmware Modification & Protocol Reference

| Field | Value |
| --- | --- |
| Document Title | Theft-Alarm LoRaWAN Node – Firmware Modification & Protocol Reference |
| Base Project | STM32WL-standalone (LoRaWAN 1.0.3 stack, originally the ANTZ Systems Temperature/Humidity sensor build) |
| Target Hardware | Seeed Wio-E5 mini / LoRa-E5 mini (STM32WLE5), digital-output (DO) vibration sensor module |
| Purpose of This Build | Theft/tamper alarm – instant confirmed uplink on vibration detection, no periodic sensor reporting |
| Status | Working – validated end-to-end (interrupt → payload → confirmed uplink → ChirpStack decode) |
| Document Date | 2026-08-28 |

This document explains, in plain language, everything that was changed in this codebase to turn a temperature/humidity LoRaWAN reference project into a working vibration-based theft alarm — and defines the resulting uplink protocol so it can be handed to anyone building the backend/dashboard. Since this firmware started life as someone else's project, treat this as the map of “what's different from the original and why.”

# 1. What This Project Actually Is Now

The starting point was ST's official STM32WL standalone LoRaWAN example, already extended by a previous author (ANTZ Systems) into a temperature/humidity sensor node: it read a sensor every 30 seconds, stored readings into flash, and uplinked them periodically (message 0xA1), plus supported a few downlink commands for configuration and battery calibration.

That periodic-sensor design doesn't fit a theft alarm at all — an alarm needs to fire the instant something happens, not on the next scheduled tick, and there's no temperature/humidity sensor connected any more. So the changes below do three things: (1) make a vibration event uplink immediately and bypass all the queuing/scheduling built for periodic sensor data, (2) turn off the now-irrelevant periodic sensor path entirely, and (3) adapt the low-level board support (UART) to the physical board actually being used (a Wio-E5 mini) instead of the ST Nucleo board the original project assumed.

# 2. Hardware Setup

| Item | Detail |
| --- | --- |
| Vibration sensor | LM393-comparator digital-output module (SW-420-style ball-switch board); onboard LED mirrors detection |
| Sensor output wiring | Sensor DO → MCU pin PB3 (board silkscreen label “A3”), interrupt line EXTI3 |
| Final GPIO setting | GPIO_MODE_IT_RISING, GPIO_NOPULL (rising edge, no internal pull – see §3.1) |
| MCU / board | STM32WLE5 on Seeed Wio-E5 mini / LoRa-E5 mini – has a built-in USB-C→UART bridge on USART1 (PB6/PB7) |
| Debug/serial console | USART1, 115200 baud, 8N1, via the board's built-in USB-C port (no external TTL adapter needed) |

One real debugging lesson worth keeping on record: partway through bring-up, the sensor's DO wire was physically plugged into a different pin than A3/PB3. That produced confusing symptoms — the sensor's own LED lit up on every tap (proving the sensor itself worked), but no uplink ever followed, while touching the bare A3 pin with a finger triggered rapid false uplinks from floating-pin noise. Once DO was correctly connected to A3, real taps produced clean, single, correct uplinks. If this ever seems to “stop working” again, re-check the physical DO→A3 connection first before suspecting the firmware.

# 3. Firmware Changes – What Was Done and Why

Five areas of the firmware were touched. Each is described below with the file(s) involved, the reasoning, and the actual code as it stands now.

## 3.1 Vibration Interrupt Configuration — gpio.c

The sensor's DO line drives the pin itself (it's a comparator output), so no internal pull resistor should fight it. After confirming (via live serial debug prints, not just multimeter guesses) that the sensor idles LOW and pulses HIGH on detection, the pin was set to trigger on the rising edge with no internal pull:

GPIO_InitStruct.Pin = VIBRATION_SENSOR_Pin; // PB3 / EXTI3 GPIO_InitStruct.Mode = GPIO_MODE_IT_RISING; // sensor pulses HIGH on detection GPIO_InitStruct.Pull = GPIO_NOPULL; // DO is push-pull; no internal pull needed

(The original project had this configured as GPIO_MODE_IT_FALLING + GPIO_PULLUP, which assumes the opposite idle-HIGH/pulse-LOW behavior – correct for some sensor variants, but not this one.)

## 3.2 Disabling the Periodic Temperature/Humidity Uplink — application_logic.c

No temperature/humidity sensor is connected in this build, so the code that used to read it, store a sample to flash, and flag a periodic uplink (message 0xA1) was disabled. Rather than deleting it, it was wrapped in a compile-time #if 0 block inside run_application_logic() — nothing is lost, and it can be switched back on later just by removing the two marker lines:

else { /* Periodic temp/humidity message (0xA1) disabled for theft-alarm build. * No temp/humidity sensor is connected; only vibration (0xA5) alerts are sent. * Re-enable by removing the #if 0 / #endif below. */ #if 0 device_status = data_sampling; turn_on_led(GREEN); read_humidity_and_temperature(&write_sensor_data.temperature, &write_sensor_data.humidity); write_sensor_data.epoch_time = SysTimeGet().Seconds; uint8_t res = store_sample(&write_sensor_data); if (1 == res) { write_sensor_data.sample_number += 1; next_action.periodic_message_flag = 1; } turn_off_led(GREEN); #endif }

Effect: the SampleTimer still ticks every 30 seconds as before, but this function is now a no-op — no sensor read, no flash write, no 0xA1 uplink ever gets queued. Vibration alerts (0xA5) and the existing downlink-triggered replies (config report, battery calibration, erase samples) are untouched and continue to work, since they run through independent code paths.

## 3.3 Debug Instrumentation — lora_app.c

To make the vibration path observable during bring-up, a debug line was added exactly where the vibration packet is sent, printed over the serial console every time a vibration-triggered uplink is attempted:

if (1 == next_action.vibration_detected) { create_vibration_message_packet(AppData.Buffer); AppData.BufferSize = VIBRATION_MESSAGE_SIZE; // 7 bytes: msg id, epoch time, vibration flag, battery uint8_t res = flush_data_onto_cloud(); sprintf(debug_buffer, "Vibration TX attempt (res=%d): %02X%02X%02X%02X%02X%02X%02X\r\n", res, AppData.Buffer[0], AppData.Buffer[1], AppData.Buffer[2], AppData.Buffer[3], AppData.Buffer[4], AppData.Buffer[5], AppData.Buffer[6]); send_data_over_uart(debug_buffer); }

res=1 means the LoRaWAN stack accepted the packet and began transmitting; the raw hex printed alongside it is the exact 7-byte payload that goes out over the air — handy for cross-checking against what ChirpStack receives.

## 3.4 UART Migration: USART2 → USART1

The original project (built for an ST Nucleo-WL55JC1 board) used USART2 on pins PA9/PA10 for its debug console, wired out to an external USB-to-serial adapter. The Wio-E5 mini / LoRa-E5 mini board used here instead has a USB-C port with a built-in USB→UART bridge wired in silicon to USART1 on PB6 (TX) / PB7 (RX) — a different physical peripheral, not just different pins. Since pin routing on this MCU isn't a free crossbar, six files needed to move from the USART2 configuration to a correct USART1 one:

| File | Change |
| --- | --- |
| Drivers/Inc/main.h | USARTx_TX_Pin / USARTx_RX_Pin macros repointed to PB6 / PB7 (were PA9 / PA10) |
| Drivers/Inc/usart.h | Declares huart1 and MX_USART1_UART_Init() (was huart2 / MX_USART2_UART_Init()) |
| Core/usart.c | Fully switched to the USART1 peripheral: correct clock source, GPIO port (GPIOB), alternate function (GPIO_AF7_USART1), DMA request (DMA_REQUEST_USART1_TX), and interrupt (USART1_IRQn) |
| Core/application_logic.c | send_data_over_uart() now initializes and transmits via huart1 instead of huart2 |
| Core/stm32wlxx_it.c / stm32wlxx_it.h | Interrupt handler renamed USART2_IRQHandler → USART1_IRQHandler, wired to huart1 |
| Core/usart_if.c | Updated for naming consistency only — this file is dead code (never called at runtime), kept just so the project still compiles cleanly |

Result: plugging the Wio-E5 mini into a computer over its USB-C port now gives a working serial console directly, at 115200-8-N-1, with no external TTL adapter required.

## 3.5 Vibration Payload Redesign — protocol.h / protocol.c / lora_app.c

The vibration message originally reused the 13-byte periodic-message layout and pulled its “epoch time” and “sample number” fields from the same flash sample-reader used by the (now-disabled) temperature/humidity path. In practice that meant a vibration alert usually carried epoch time = 0 and sample number = 0, because no periodic sample happened to be pending in flash at that instant — the timestamp was borrowed data, not a real capture of when the vibration actually happened.

Since knowing exactly when a theft alert fired matters, this was replaced with a compact, purpose-built 7-byte format that stamps the real time at the moment of the event, using the LoRaWAN stack's own system clock (SysTimeGet()) directly — no dependency on the flash sample queue at all:

// protocol.h #define VIBRATION_MESSAGE_SIZE 7 // protocol.c void create_vibration_message_packet(uint8_t *buf) { memset(buf, 0, LORAWAN_APP_DATA_BUFFER_MAX_SIZE); uint32_t epoch_now = SysTimeGet().Seconds; buf[0] = vibration_message; // 0xA5 /* bytes 1-4: live epoch time at the moment of this vibration event */ buf[1] = (epoch_now & 0xFF000000) >> 24; buf[2] = (epoch_now & 0x00FF0000) >> 16; buf[3] = (epoch_now & 0x0000FF00) >> 8; buf[4] = epoch_now & 0xFF; /* byte 5: vibration flag (1 = vibration detected) */ buf[5] = 0x01; /* byte 6: battery percentage */ buf[6] = get_battery_percentage(); }

lora_app.c's send path was updated to match — AppData.BufferSize now uses the new VIBRATION_MESSAGE_SIZE (7) constant instead of the old 13-byte PERIODIC_MESSAGE_SIZE, and the debug print (§3.3) was trimmed to print 7 bytes instead of 13.

## 3.6 Summary – All Files Touched

| File | What Changed |
| --- | --- |
| Core/gpio.c | Vibration pin (PB3/EXTI3): rising edge, no internal pull |
| Core/application_logic.c | Periodic temp/humidity sampling disabled (#if 0); UART calls repointed to huart1 |
| LoRaWAN/App/lora_app.c | Vibration TX debug print added; buffer size switched to the new 7-byte format |
| Drivers/Inc/protocol.h | Added VIBRATION_MESSAGE_SIZE (7) |
| Core/protocol.c | create_vibration_message_packet() rewritten: live epoch timestamp, no flash dependency |
| Drivers/Inc/main.h | UART pin macros repointed to PB6/PB7 |
| Drivers/Inc/usart.h | Declares huart1 / MX_USART1_UART_Init() |
| Core/usart.c | Switched from USART2 to USART1 peripheral (clock, GPIO AF, DMA, IRQ) |
| Core/stm32wlxx_it.c / .h | USART interrupt handler renamed to USART1_IRQHandler |
| Core/usart_if.c | Renamed for consistency (unused/dead code, doesn't affect runtime) |

Not a git repository, so no formal diff/patch exists — the snippets above are the authoritative record of what changed.

# 4. Uplink/Downlink Protocol – Current State

This section documents the protocol as it actually behaves in this build, extending the original project's protocol spec with the new vibration message and flagging what's now inactive.

## 4.1 Vibration Alert — 0xA5 (new, primary message for this project)

Size: 7 bytes

Trigger: Immediately on the rising edge of the vibration sensor's DO output (EXTI3 interrupt) — not on a timer, and not gated by link-check status.

Confirmation: Sent as a confirmed LoRaWAN uplink (ACK requested from the network server).

| Byte | Field | Type | Encoding |
| --- | --- | --- | --- |
| 0 | Message ID | uint8 | 0xA5 (vibration_message) |
| 1–4 | Epoch Time | uint32 BE | Unix timestamp (seconds), captured live via SysTimeGet() at the moment of the event |
| 5 | Vibration Flag | uint8 | 0x01 = vibration detected |
| 6 | Battery | uint8 | 0–100 (%) |

### Example

Hex payload: A5 6A9132C4 01 64

| Field | Hex | Decoded |
| --- | --- | --- |
| Message ID | A5 | 0xA5 (Vibration Alert) |
| Epoch Time | 6A9132C4 | 1787900612 → 2026-08-28 12:33:32 IST |
| Vibration Flag | 01 | Vibration detected |
| Battery | 64 | 100% |

## 4.2 Messages Carried Over From the Original Project

These still exist in the code and remain functional, but their relevance has changed now that there's no temperature/humidity sensor in this build:

| Msg | Name | Size | Status in This Build |
| --- | --- | --- | --- |
| 0xA1 | Periodic Sensor Data | 13 bytes | Disabled — code path is compiled out (§3.2); no longer sent |
| 0xA2 | Device Configuration Report | 8 bytes | Still functional (responds to 0xB1/0xB2), but sample_interval reported is now meaningless since sampling is off |
| 0xA3 | Battery Calibration ACK | 2 bytes | Still functional and useful — unrelated to sensor sampling |
| 0xA4 | Erase Samples ACK | 2 bytes | Still functional, though the flash sample buffer it clears is no longer being written to |
| 0xB1 | Update Sampling Interval (downlink) | 3 bytes | Still accepted, but has no visible effect since sampling is disabled |
| 0xB2 | Fetch Device Configuration (downlink) | 1 byte | Still functional |
| 0xB3 | Battery Calibration (downlink) | 2 bytes | Still functional |
| 0xB4 | Erase Pending Samples (downlink) | 1 byte | Still functional |

Recommendation: if 0xA1/0xB1 aren't going to be revisited, it's worth deciding whether to keep them wired up (no harm, just unused) or strip them later for a cleaner build — not urgent either way.

## 4.3 Application-Layer Clock Synchronization (Port 202)

Inherited from the original project: a ClockSyncTimer fires 10 seconds after boot, then every 8 hours, and runs the standard LoRaWAN Application Layer Clock Synchronization package (AppTimeReq/AppTimeAns) on dedicated port 202. This is how the device's epoch clock (used in the 0xA5 payload) gets corrected against the network's time — not GPS, not a manually-set RTC.

Caveat worth tracking: this correction only lands if the AppTimeAns downlink is actually received, which depends on the same RF link used for confirmed-uplink ACKs. A weak link (see §5) can cause this sync to silently fail, leaving the device coasting on a stale correction — which is consistent with a small (~18–19 second) constant timestamp offset observed during testing.

# 5. Known Issue – RF Link Margin

During testing, ChirpStack's LinkCheckAns (the network server's answer to the device's automatic link-check, triggered after a failed confirmed-uplink ACK) reported a demodulation margin of 0 dB. That means the uplink was received right at the edge of decodability — no headroom. This is consistent with the repeated “No ACK Received. Starting link check recovery.” and “Message Send Fail” lines seen in the debug log: the device is transmitting correctly, ChirpStack is receiving the uplinks and does attempt the ACK, but the return path (gateway → device) is marginal enough that the device often doesn't hear it.

This isn't a firmware defect — it's a physical-layer/range issue. It's worth keeping in mind because it affects two things beyond just “noisy logs”: confirmed-uplink reliability, and (per §4.3) the reliability of the periodic clock re-sync. Options if it needs to be tightened up: improve antenna placement/line-of-sight to the gateway, or force a lower data rate (more spreading factor = more sensitivity margin, at the cost of airtime).

# 6. Network-Side: ChirpStack Payload Decoder

On the ChirpStack side (Device Profile → Codec tab → Custom JavaScript codec), a decodeUplink() function was added so the dashboard/API sees a clean decoded object instead of raw hex on every uplink:

function decodeUplink(input) { var bytes = input.bytes; var msgId = bytes[0]; function toEpoch(b1, b2, b3, b4) { return ((b1 << 24) ¦ (b2 << 16) ¦ (b3 << 8) ¦ b4) >>> 0; } function toIST(epochSeconds) { var utcMs = epochSeconds * 1000; var istMs = utcMs + (5 * 60 + 30) * 60 * 1000; // UTC+5:30 var d = new Date(istMs); var pad = function (n) { return (n < 10 ? "0" : "") + n; }; return d.getUTCFullYear() + "-" + pad(d.getUTCMonth() + 1) + "-" + pad(d.getUTCDate()) + " " + pad(d.getUTCHours()) + ":" + pad(d.getUTCMinutes()) + ":" + pad(d.getUTCSeconds()) + " IST"; } if (msgId === 0xA5 && bytes.length === 7) { var epoch = toEpoch(bytes[1], bytes[2], bytes[3], bytes[4]); return { data: { message_type: "Vibration Alert", alert: bytes[5] === 1 ? "Vibration detected!" : "No vibration", event_time_ist: toIST(epoch), event_time_utc: new Date(epoch * 1000).toISOString(), battery_percent: bytes[6] } }; } if (msgId === 0xA2 && bytes.length === 8) { var batteryCounter = toEpoch(bytes[1], bytes[2], bytes[3], bytes[4]); return { data: { message_type: "Device Config Report", battery_counter: batteryCounter, sample_interval_sec: (bytes[5] << 8) ¦ bytes[6], battery_percent: bytes[7] } }; } if (msgId === 0xA3 && bytes.length === 2) { return { data: { message_type: "Battery Calibration ACK", battery_percent: bytes[1] } }; } if (msgId === 0xA4 && bytes.length === 2) { return { data: { message_type: "Erase Samples ACK", battery_percent: bytes[1] } }; } return { data: { message_type: "Unknown", raw_hex: bytes.map(function (b) { return ("0" + b.toString(16)).slice(-2); }).join("").toUpperCase() } }; }

For a vibration alert, the decoded object looks like this (shown in ChirpStack's event detail view, and delivered the same way to any app/dashboard reading via the API or MQTT integration):

{ "message_type": "Vibration Alert", "alert": "Vibration detected!", "event_time_ist": "2026-08-28 12:33:32 IST", "event_time_utc": "2026-08-28T07:03:12.000Z", "battery_percent": 100 }

Note: this codec lives only on the ChirpStack server — it's not part of the firmware, and changing it requires no reflash, just editing it in the Device Profile's Codec tab.

# 7. Quick Reference

## 7.1 Pin Map

| Signal | Pin | Notes |
| --- | --- | --- |
| Vibration sensor DO | PB3 (silkscreen “A3”) | EXTI3, rising edge, no pull |
| Debug UART TX | PB6 | USART1, via built-in USB-C bridge |
| Debug UART RX | PB7 | USART1, via built-in USB-C bridge |

## 7.2 Message ID Map

| ID | Direction | Name | Active in This Build? |
| --- | --- | --- | --- |
| 0xA1 | Uplink | Periodic Sensor Data | No – disabled |
| 0xA2 | Uplink | Device Configuration Report | Yes |
| 0xA3 | Uplink | Battery Calibration ACK | Yes |
| 0xA4 | Uplink | Erase Samples ACK | Yes |
| 0xA5 | Uplink | Vibration Alert | Yes – primary message |
| 0xB1 | Downlink | Update Sampling Interval | Accepted, no effect |
| 0xB2 | Downlink | Fetch Device Configuration | Yes |
| 0xB3 | Downlink | Battery Calibration | Yes |
| 0xB4 | Downlink | Erase Pending Samples | Yes |